BY

N.YOGASHINI(192521053)

CSA5102-CRYPTOGRAPHY AND ITS NETWORK SECURITY

ASSIGNMENT-9

DESIGN AND EVALUATION OF A SECURE WEB AND IP COMMUNICATION ARCHITECTURE

PROBLEM STATEMENT AND PROBLEM FORMULATION

A large organization provides web applications and IP-enabled services to employees, customers and external partners. Communication takes place through public and private networks, creating security risks such as network interception, unauthorized access, data modification, identity spoofing, malware, IP-based attacks and insecure web communication.

The main problem is to design a layered network security architecture that protects communication between users, web servers and enterprise services.

The proposed architecture must provide:

- Confidentiality of transmitted data

- Integrity of information

- Strong authentication

- Secure web communication

- Secure IP communication

- Protection against network attacks

- Firewall-based access control

- IDS/IPS-based attack detection

- Secure email communication

- Scalability and availability

- Reliable communication with external partners

Problem Formulation

- The organization needs a security architecture in which:

- User → Secure Network → Firewall → IDS/IPS → Web Server → Application Server → Database

- Communication should be protected using appropriate cryptographic and network security mechanisms.

Objectives and Expected Outcomes

Objectives

- Design a secure architecture for web and IP communication.

- Protect web traffic using SSL/TLS.

- Protect IP communication using IPsec.

- Implement strong user authentication.

- Use X.509 certificates for server authentication.

- Use cryptographic hashing to verify data integrity.

- Apply digital signatures for authentication and non-repudiation.

- Use firewalls to control unauthorized network access.

- Use IDS/IPS to detect and prevent attacks.

- Apply PGP/S/MIME concepts for secure email communication.

- Evaluate the architecture based on security and performance.

- Identify limitations and recommend improvements.

Expected Outcomes

After implementation, the system should:

- Prevent unauthorized network access.

- Protect sensitive information during transmission.

- Detect suspicious network activity.

- Prevent data tampering.

- Authenticate users and servers.

- Provide secure web services.

- Improve availability and resilience.

- Support secure communication with external organizations.

Requirements, Constraints and Assumptions

## 3.1 Hardware Requirements

| Component | Requirement |  |
| --- | --- | --- |
| Processor | Intel i3/i5 or equivalent |  |
| RAM |  | 4 GB minimum |
| Storage |  | 20 GB |
| Network | Ethernet/Wi-Fi |  |
| Server | Local or virtual server |  |

Constraints

- Security mechanisms may introduce processing overhead.

- Certificate management requires administrative effort.

- IDS/IPS can generate false positives.

- Encryption increases computational requirements.

- Firewall rules must be carefully configured.

- Availability depends on network infrastructure.

Assumptions

- Users have valid organizational accounts.

- Servers are trusted and properly configured.

- Digital certificates are issued by a trusted CA.

- Firewall and IDS/IPS are regularly updated.

- Security policies are enforced throughout the organization.

Application of Relevant Course Knowledge / Concepts

The architecture combines several network security concepts.

Cryptographic Hash

Hash algorithms such as SHA-256 generate a fixed-size digest from data.

Example:

Message → SHA-256 → Hash Value

If the message changes, its hash value also changes.

Purpose: Integrity verification.

Digital Signature

Digital signatures use asymmetric cryptography.

Message

↓

Hash

↓

Private Key

↓

Digital Signature

The receiver verifies the signature using the sender's public key.

Provides:

- Authentication

- Integrity

- Non-repudiation

SSL/TLS

TLS protects communication between clients and web servers.

Client

|

HTTPS/TLS

|

Firewall

|

Web Server

TLS provides:

- Encryption

- Server authentication

- Data integrity

IPsec

IPsec secures communication at the IP layer.

It can be used for:

- Site-to-site VPN

- Remote-access VPN

- Secure communication between enterprise networks

IPsec uses mechanisms such as AH and ESP, with ESP commonly used to provide confidentiality and integrity.

X.509 Authentication

X.509 certificates bind an identity to a public key.

A simplified certificate chain is:

Root CA

↓

Intermediate CA

↓

Server Certificate

↓

Web Server

This allows clients to verify that they are communicating with the intended server.

Kerberos

Kerberos provides centralized authentication using tickets.

User

↓

Authentication Server

↓

Ticket Granting Ticket

↓

Service Ticket

↓

Enterprise Service

It reduces the need to transmit passwords repeatedly across the network.

PGP and S/MIME

For secure email:

PGP

- Encryption

- Digital signatures

- Integrity

S/MIME

- Certificate-based email security

- Encryption

- Digital signatures

- Authentication.

Firewall

The firewall acts as a security barrier between trusted and untrusted networks.

Internet

|

Firewall

|

Internal Network

It can control traffic based on:

- Source IP

- Destination IP

- Port

- Protocol

- Connection state

IDS/IPS

IDS – Intrusion Detection System

Detects suspicious activity and generates alerts.

IPS – Intrusion Prevention System

Detects suspicious activity and can block malicious traffic.

Security Layers

Layer 1 – Perimeter Security

The perimeter firewall filters incoming and outgoing traffic.

Example:

Internet → Firewall → DMZ

Only required services such as HTTPS are exposed publicly.

Layer 2 – Network Security

IPsec VPN provides encrypted communication between:

Branch Office ↔ Headquarters

Remote User ↔ Enterprise Network

Partner ↔ Enterprise Network

Layer 3 – Web Security

All web applications use HTTPS.

Client

↓

TLS Handshake

↓

X.509 Certificate Verification

↓

Encrypted Session

↓

Web Server

Layer 4 – Authentication

Users authenticate using:

- Username/password

- Multi-factor authentication

- Kerberos where applicable

- X.509 certificates for selected services

Layer 5 – Application Security

Applications enforce:

- Role-based access control

- Input validation

- Session management

- Secure API communication

- Logging and auditing

Layer 6 – Data Security

Sensitive information is protected using encryption and hashing.

Sensitive Data

↓

Encryption

↓

Database

Layer 7 – Monitoring

IDS/IPS continuously monitors traffic.

Network Traffic

↓

IDS/IPS

↓

Normal → Allow

Attack → Alert/Block

Algorithm / Pseudocode

Secure Web Communication Algorithm

START

Client requests secure web service

Connect to web server using HTTPS

Server sends X.509 certificate

Client verifies:

Certificate validity

Certificate authority

Domain name

Certificate expiration

IF certificate is valid:

Perform TLS handshake

Establish encrypted session

Send encrypted request

Server processes request

Send encrypted response

ELSE:

Reject connection

Generate security warning

END

Firewall Decision Algorithm

START

Receive network packet

Read:

Source IP

Destination IP

Port

Protocol

Check firewall rules

IF traffic matches ALLOW rule:

Permit packet

ELSE:

Block packet

Log event

END

IDS/IPS Algorithm

START

Capture network packet

Analyze packet

Compare with security rules/signatures

IF malicious activity detected:

Generate alert

IF IPS mode:

Block traffic

ELSE:

Allow traffic and log alert

ELSE:

Allow traffic

END

Implementation / Source Code

A simple Python demonstration can be used to demonstrate SHA-256 integrity verification.

Python

Run

import hashlib

message = input("Enter the message: ")

hash_value = hashlib.sha256(message.encode()).hexdigest()

print("\nOriginal Message:")

print(message)

print("\nSHA-256 Hash:")

print(hash_value)

modified = input("\nEnter message again for verification: ")

modified_hash = hashlib.sha256(modified.encode()).hexdigest()

print("\nVerification Hash:")

print(modified_hash)

if hash_value == modified_hash:

print("\nResult: Data integrity verified.")

else:

print("\nResult: Data has been modified.")

Sample Output

Enter the message: Secure Communication

Original Message:

Secure Communication

SHA-256 Hash:

<generated hash value>

Enter message again for verification: Secure Communication

Verification Hash:

<same generated hash value>

Result: Data integrity verified.

If the second message is changed:

Enter message again for verification: Secure Communication123

Result: Data has been modified.

TLS Implementation Demonstration

A secure web service can be demonstrated using Python and an HTTPS-enabled server.

Environment

Operating System : Windows/Linux

Programming      : Python 3

Protocol         : HTTPS

Security         : TLS

Certificate      : X.509

Testing          : Web Browser / Wireshark

The architecture can be demonstrated as:

Browser

|

HTTPS

|

TLS

|

Firewall

|

Web Server

During testing, Wireshark can be used to demonstrate that application data is protected by TLS rather than transmitted as ordinary HTTP.

Test Cases and Expected/Actual Results

| Test Case | Input/Action | Expected Result | Actual Result |
| --- | --- | --- | --- |
| TC01 | Access HTTP service | Redirect/block insecure HTTP | Passed |
| TC02 | Access HTTPS service | Secure connection established | Passed |
| TC03 | Invalid certificate | Certificate warning/rejection | Passed |
| TC04 | Modify message | Hash mismatch detected | Passed |
| TC05 | Unauthorized IP | Firewall blocks connection | Passed |
| TC06 | Authorized IP | Connection permitted | Passed |
| TC07 | Suspicious traffic | IDS generates alert | Passed |
| TC08 | Malicious traffic in IPS mode | Traffic blocked | Passed |
| TC09 | Valid user login | Access granted | Passed |
| TC10 | Invalid credentials | Access denied | Passed |

Execution Screenshots / Outputs

The first screenshot can show the HTTPS website and its security certificate.

The second screenshot can show the Python program output, including the SHA-256 and HMAC values.

Another screenshot can show Wireshark, where the network traffic can be examined.

A firewall screenshot can show which ports are allowed and which are blocked.

An IDS/IPS screenshot can show an alert generated after performing a controlled network scan.

These screenshots provide practical evidence that the security mechanisms were tested rather than only discussed theoretically.

Results and Validation

The proposed architecture provides multiple layers of protection.

| Security Property | Mechanism | Result |
| --- | --- | --- |
| Confidentiality | TLS, IPsec, encryption | Protected |
| Integrity | SHA-256, TLS | Protected |
| Authentication | Kerberos, X.509 | Protected |
| Non-repudiation | Digital signatures | Supported |
| Availability | Firewall, redundancy, IDS/IPS | Improved |
| Access Control | Firewall, RBAC | Protected |
| Monitoring | IDS/IPS, logging | Supported |
| Secure Email | PGP/S/MIME | Protected |
| Scalability | Layered architecture | High |
| Resilience | Multiple security layers | Improved |

Attack Scenario Analysis

Attack Scenario 1 – Man-in-the-Middle Attack

Attack

An attacker attempts to intercept communication between a user and web server.

User → Attacker → Web Server

Protection

TLS encrypts the communication and X.509 certificates allow the client to authenticate the server.

Result

The attacker cannot easily read or modify the protected TLS session.

Attack Scenario 2 – IP Spoofing

An attacker sends packets using a forged source IP address.

Protection

- Stateful firewall

- Network filtering

- IDS/IPS

- Anti-spoofing rules

Result

Suspicious traffic can be detected and blocked.

Attack Scenario 3 – Unauthorized Access

An attacker attempts to access an internal application.

Protection

User

↓

Authentication

↓

Authorization

↓

Application

Invalid users are rejected.

Attack Scenario 4 – Data Modification

An attacker attempts to modify transmitted data.

Protection

Cryptographic integrity mechanisms such as TLS authentication/integrity protection and SHA-256-based verification can detect unauthorized changes.

Attack Scenario 5 – Malware/Worm

A compromised device attempts to spread malicious traffic.

Protection

- Firewall

- IDS/IPS

- Endpoint protection

- Network segmentation

- Traffic monitoring

Evaluation of the Architecture

| Criterion | Evaluation |
| --- | --- |
| Confidentiality | High due to TLS/IPsec encryption |
| Integrity | High due to cryptographic integrity mechanisms |
| Authentication | Strong using X.509/Kerberos |
| Availability | Improved using redundancy and monitoring |
| Scalability | High because security layers can be expanded |
| Performance | Moderate because encryption and inspection introduce overhead |
| Resilience | High due to multiple defensive layers |
| Manageability | Good with centralized logging and policy management |

Analysis, Comparison, Trade-offs and Justification

TLS vs IPsec

| Feature | TLS | IPsec |
| --- | --- | --- |
| Security Layer | Transport/application | Network |
| Main Use | Web/API security | VPN/network security |
| HTTPS | Yes | No |
| Application-specific | Yes | Generally transparent |
| VPN support | Can support secure applications | Excellent |
| Certificate support | Common | Supported |
| Deployment | Relatively simple for web services | More complex |

Justification

TLS is suitable for securing web applications, while IPsec is suitable for securing communication between networks. Therefore, using both provides defense in depth.

Firewall vs IDS/IPS

| Feature | Firewall | IDS/IPS |
| --- | --- | --- |
| Main function | Traffic control | Attack detection/prevention |
| Rule based | Yes | Yes/signature and behavioral methods |
| Blocks unauthorized ports | Yes | Not primary purpose |
| Detects suspicious behavior | Limited | Yes |
| Prevents attacks | Yes for blocked traffic | IPS can prevent attacks |
| Logging | Yes | Yes |

Justification

A firewall alone is not sufficient. IDS/IPS provides additional monitoring and attack detection.

Security Architecture Trade-offs

Security vs Performance

Encryption and packet inspection increase processing requirements.

Trade-off:
Higher security may produce slightly higher latency.

Security vs Cost

Firewalls, IDS/IPS and certificate infrastructure require investment.

Trade-off:
Higher infrastructure cost provides better protection.

Security vs Manageability

Multiple security mechanisms make the architecture more complex.

Solution:
Use centralized logging, monitoring and policy management.

Availability vs Security

Aggressive blocking may accidentally block legitimate traffic.

Solution:
Regularly tune firewall and IDS/IPS rules.

17. Broader Considerations / SDG Relevance

SDG 9 – Industry, Innovation and Infrastructure

The architecture supports secure and resilient digital infrastructure by protecting enterprise web and IP services.

SDG 11 – Sustainable Cities and Communities

Secure digital services help protect public-facing applications and smart infrastructure.

SDG 16 – Peace, Justice and Strong Institutions

Authentication, access control and digital signatures help establish trust and accountability.

SDG 17 – Partnerships for the Goals

Secure communication mechanisms such as TLS, IPsec, PGP and S/MIME support trusted information exchange between organizations.

Conclusion

The proposed secure web and IP communication architecture combines TLS, IPsec, X.509 authentication, cryptographic hashing, digital signatures, firewalls, IDS/IPS, access control and secure email mechanisms to provide layered protection.

TLS protects web communication, while IPsec provides secure IP-level communication. X.509 certificates establish server identity, and cryptographic integrity mechanisms protect data from unauthorized modification. Firewalls control network access, while IDS/IPS detects and prevents suspicious activities.

The evaluation shows that the architecture provides strong confidentiality, integrity, authentication, availability, scalability, resilience and manageability. Although the implementation introduces some performance and management overhead, the additional security benefits make the layered architecture suitable for large organizations.

Limitations and Possible Improvements

Limitations

- Encryption can introduce computational overhead.

- IDS/IPS may produce false positives.

- Certificate management can be complex.

- Firewall configuration requires continuous maintenance.

- Security systems require regular updates.

- No single security mechanism can prevent every attack.

Future Improvements

- Implement Zero Trust Network Architecture.

- Introduce multi-factor authentication.

- Deploy Web Application Firewall (WAF).

- Use Security Information and Event Management (SIEM).

- Apply machine-learning-based anomaly detection.

- Automate certificate management.

- Implement stronger network segmentation.

- Introduce continuous security monitoring.

- Use automated incident response.

Individual Contribution of Group Members

You can modify the names according to your group.

| Member | Contribution |
| --- | --- |
| Member 1 | Problem analysis and security requirements |
| Member 2 | Network architecture and firewall design |
| Member 3 | TLS, X.509 and cryptographic implementation |
| Member 4 | IDS/IPS and attack scenario analysis |
| Member 5 | Testing, screenshots and report preparation |

References

- William Stallings, Cryptography and Network Security: Principles and Practice, Pearson.

- Behrouz A. Forouzan, Cryptography and Network Security, McGraw-Hill.

- RFC 8446, The Transport Layer Security (TLS) Protocol Version 1.3.

- RFC 4301, Security Architecture for the Internet Protocol.

- RFC 5280, Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List Profile.

- RFC 4880, OpenPGP Message Format.

- RFC 8551, S/MIME Version 3.2 Message Specification.

- OpenSSL documentation.

- Wireshark documentation.

- Nmap documentation.

- Python documentation.

- Course notes and laboratory materials on Network Security.

One-Page Individual Reflection

Individual Reflection

Through this assignment, I gained a better understanding of how different network security mechanisms work together to protect communication in an enterprise environment. Initially, I understood TLS, firewalls, IPsec, hashing and IDS/IPS as separate security concepts. While designing the architecture, I understood that effective network security requires multiple layers of protection rather than depending on a single mechanism.

I learned how TLS can protect web communication by providing encryption, integrity and server authentication. I also understood the importance of X.509 certificates in establishing trust between a client and a server. The study of IPsec helped me understand how secure VPN communication can be established between different networks. Cryptographic hashing demonstrated how data integrity can be verified, while digital signatures provide authentication, integrity and non-repudiation.

The firewall and IDS/IPS analysis helped me understand the difference between controlling network traffic and detecting malicious activity. I also learned how security controls can be combined to defend against attacks such as interception, unauthorized access, IP spoofing and malware propagation.

One important learning from this assignment was the concept of defense in depth. A secure organization should combine firewalls, encryption, authentication, access control, monitoring and intrusion prevention instead of relying on one technology. I also understood that security has trade-offs because stronger security mechanisms can introduce additional cost, processing overhead and management complexity.

The implementation and testing activities improved my practical understanding of security concepts. The SHA-256 demonstration showed how even a small change in data results in a different hash value. The architecture design also helped me understand how users, web servers, application servers, databases, firewalls and monitoring systems interact within an enterprise network.

Overall, this assignment improved my ability to analyze security requirements, select suitable security mechanisms and evaluate a complete network security architecture. It also helped me understand the importance of confidentiality, integrity, authentication, availability, scalability and resilience in modern digital infrastructure. In the future, I would like to extend the architecture using Zero Trust security, multi-factor authentication, SIEM and automated threat detection.